# Hydrogen Storage Vessel Simulation

## Data & Model Specification (Final)

Prepared for: Abdul Yunusa Ruwani — BSc Thesis, Petroleum & Gas Engineering, University of Lagos

Purpose: Predict H2 adsorption isotherm behavior and full vessel charge/discharge performance for a Ca-exchanged Zeolite-14A hydrogen storage system. No physical H2 testing has been performed — this is a fully predictive simulation to generate the data the thesis needs.

# 1. System Description

- Adsorbent: Zeolite-14A, tested in 4 sample configurations (2×2 design):

- Activated (Ca²⁺ ion-exchanged) powder

- Regular (untreated) powder

- Activated (Ca²⁺ ion-exchanged) granule

- Regular (untreated) granule

- Ca-exchange performed via CaCl2 treatment, thermal activation at 300°C

- Application: portable cryogenic H2 storage vessel (physical prototype fabricated, design available on request)

- Benchmark: US DOE gravimetric target = 5.5 wt%

# 2. Adsorbent & Isotherm Data

- Isotherm model: Langmuir

- Saturation capacity (qmax): not experimentally fixed — model to predict maximum achievable capacity for each of the 4 samples.

Developer note: for plausibility-checking model output, published data on cation-exchanged zeolites (CaX, KX) at near-ambient temperature and 0–15 bar report gravimetric capacities up to ~2.2 wt% for the best-performing framework/cation combination; zeolite-A-type frameworks are expected to underperform X/Y-type frameworks at equivalent conditions due to pore-blocking by extraframework cations. At the cryogenic temperature specified below, higher capacities than these near-ambient benchmarks are physically expected — treat these figures as a sanity-check floor, not a ceiling.

- Operating/simulation temperature: −78°C (cryogenic, held via external dry ice/acetone bath — locked value)

- Pressure range: 1–10 bar

- Isosteric heat of adsorption (ΔH_ads): use literature default for cation-exchanged zeolite/H2 physisorption (~5–8 kJ/mol) — not experimentally measured

- Experimental data points: none — fully predictive model

- Samples modeled separately: yes, all 4 (activated powder, regular powder, activated granule, regular granule)

# 3. Vessel Physical Specification

- Configuration: single vessel, reused sequentially for each of the 4 sample runs (125 g per sample)

- Geometry: cylindrical, 50 mm diameter × 250 mm length

- Wall material: stainless steel

- Wall thickness: 10 mm

- Bed containment: mesh filters

- Bed void fraction: 0.35–0.40

- Insulation: present, to prevent heat loss from the cryogenic system (type/thickness not specified — developer to assume standard cryogenic insulation practice, e.g. vacuum jacket or foam wrap, and flag the assumption)

- Cooling method: external dry ice/acetone bath (~−78°C)

- Max design/rated pressure: 15 bar

Vessel CAD/design file available from Abdul on request.

# 4. Operating Conditions & Duty Cycle

- Charging source pressure: below 10 bar

- Target charge pressure: 10 bar

- Charging temperature: −78°C (same cryogenic bath as loading)

- Charge rate/duration: not fixed — left to the simulation to determine

- Desorption trigger: heating to 80–100°C (confirmed — not ambient release)

- Discharge delivery requirement: intended for fuel cell feed (constant/regulated flow assumed unless developer has a specific fuel cell demand curve to target)

- Number of cycles to simulate: 5

- Ambient/surroundings temperature: 25°C

# 5. Gas-Phase & Thermodynamic Modeling Choices

- Isothermal model: confirmed (external cold bath assumed to hold constant temperature throughout charge/discharge)

- Equation of state: ideal gas (adequate at 1–15 bar)

- Adsorption/desorption kinetics: instantaneous equilibrium (not rate-limited)

# 6. Numerical / Software Setup

- Platform: developer's choice

- Time resolution: minutes

- Spatial resolution: lumped, well-mixed tank model (no 1D bed profile)

# 7. Required Outputs

- Isotherm plot for all 4 samples — adsorption capacity (wt%) vs. partial pressure

- Desorption vs. time

- Mass of hydrogen adsorbed per sample

Recommended addition, not explicitly requested but implied by the duty cycle above: vessel pressure and H2 mass stored vs. time during charge, and a capacity-vs-cycle-number plot across the 5 cycles, for completeness against the degradation testing described elsewhere in the thesis — confirm with Abdul if these should be included.

# 8. Resolved Items

- DOE target ambiguity → 5.5 wt% confirmed

- Loading/charging temperature ambiguity → −78°C confirmed (dry ice/acetone bath)

- Desorption conditions → 80–100°C heating required confirmed

- Material naming → Zeolite-14A (both powder and granule forms)

- Bed void fraction → 0.35–0.40 confirmed